# Smart Environment Monitoring and Control System

SWAPD 453 - IoT Applications | Spring 2026

| Team Name | SmartPulse |
| --- | --- |
| Member 1 | Rana Hazem - 202201938 |
| Member 2 | Elaliaa Elmoez - 202402473 |
| Implementation | ESP32 + MicroPython on Wokwi, MQTT, Node-RED Dashboard |

This report summarizes the implemented system, current project status, challenges faced, and the remaining planned extensions for final submission.

# 1. Introduction

This project presents a Smart Environment Monitoring and Control System built using ESP32, multiple sensors, actuators, MQTT communication, and a Node-RED dashboard. The system monitors environmental conditions in real time and provides manual control over connected devices through a web dashboard.

The project simulates a smart room or smart lab scenario. Sensor data is collected using the ESP32, published through MQTT topics, and visualized on a Node-RED dashboard. Control commands travel in the opposite direction from the dashboard to the ESP32 to operate actuators such as LEDs, buzzer, relay, and servo motor.

# 2. System Description

The system is composed of three main parts:

1. ESP32 node: reads sensors, publishes values, subscribes to control topics, and executes actuator commands.
2. MQTT communication layer: transfers data between the ESP32 and Node-RED using a publish/subscribe model.
3. Node-RED dashboard: shows live sensor readings and provides manual control widgets for actuators.

This structure creates a complete IoT loop: sensing, communication, monitoring, and control.

# 3. Components Used

## Hardware / Simulated Components

ESP32 Dev Module

DHT22 temperature and humidity sensor

LDR / photoresistor module

PIR motion sensor

HC-SR04 ultrasonic sensor

Red, green, and yellow LEDs

Buzzer

Relay module

Servo motor

## Software / Platforms

Wokwi simulator

MicroPython

MQTT broker

Node-RED dashboard

# 4. System Architecture

The overall architecture works as follows: sensors connected to the ESP32 read the environment continuously, the ESP32 publishes these values to MQTT topics, Node-RED subscribes to the topics and displays them on a dashboard, and user actions from the dashboard are sent back through MQTT to control the actuators.

Data flow: Sensors -> ESP32 -> MQTT Broker -> Node-RED Dashboard, and Node-RED Dashboard -> MQTT Broker -> ESP32 -> Actuators.

# 5. Implemented Features

## 5.1 Sensor Monitoring

Temperature

Humidity

Light intensity

Motion status

Distance

Each sensor is connected to the ESP32 and is read periodically. The values are published through MQTT and displayed live on the dashboard.

## 5.2 Dashboard Visualization

Gauge for temperature

Gauge for humidity

Gauge for light level

Gauge for distance

Text/status display for motion detection

## 5.3 Manual Control

LED control using dashboard switch

Buzzer control using dashboard switch

Relay control using dashboard switch

Servo angle control using dashboard slider

# 6. MQTT Topics Used

## Sensor Topics

rana7/iot/temperature

rana7/iot/humidity

rana7/iot/light

rana7/iot/motion

rana7/iot/distance

## Control Topics

rana7/iot/control/led

rana7/iot/control/buzzer

rana7/iot/control/relay

rana7/iot/control/servo

# 7. Current Dashboard Status

The monitoring section currently displays temperature, humidity, light, distance, and motion values in real time.

The controls section currently provides manual switches for LED, buzzer, and relay, in addition to a slider for servo angle control.

The dashboard was verified using live values received from the MQTT broker.

# 8. Challenges Faced

MQTT connectivity required multiple trials because local and public brokers behaved differently in simulation and on the laptop.

Node-RED dashboard widgets required careful configuration of groups, tabs, gauges, and text nodes before values were displayed correctly.

Dashboard switch nodes sometimes sent boolean values (true/false) instead of numeric values (1/0), which required a function node to normalize payloads.

Matching the exact GPIO pins in Wokwi with the code was important, especially for LEDs, buzzer, relay, and servo motor.

Servo control required proper PWM frequency and duty mapping to move correctly across the requested angle range.

# 9. Lessons Learned

How to integrate multiple sensors and actuators with ESP32.

How to use MQTT publish/subscribe communication in an IoT system.

How to create a live dashboard using Node-RED.

How to troubleshoot integration issues between embedded code and dashboard systems.

How to build a complete end-to-end IoT workflow from sensing to control.

# 10. Remaining Planned Work

The following items are planned to complete and polish the final submission:

Add and refine auto-control logic based on threshold values.

Add threshold adjustment widgets from the dashboard if required.

Export the complete Node-RED flow JSON file.

Capture final Wokwi wiring screenshots and dashboard screenshots.

Prepare the final ZIP package and GitHub repository structure.

# 11. Conclusion

This project successfully implemented a Smart Environment Monitoring and Control System using ESP32, sensors, actuators, MQTT, and Node-RED. The system can monitor environmental conditions in real time and provides a user-friendly dashboard for visualization and manual control.

The current implementation already demonstrates a complete IoT workflow and provides a strong foundation for future additions such as threshold-based automation, alerts, and logging.

# 12. GitHub Repository

https://github.com/rana7azem/SmartPulse_SmartEnv.git